Code description

# Introduction

The aim of this document is to put together information from several models that were applied to simulate particular study cases in order to understand the differences resulting from their inter-comparison. The cases studied were isothermal in two or three dimensions. A requirement is this document to be as simple and as clear as possible. For this reason and also due to the large amount of information that can be put in, a main table (code description) and two additional tables namely, general equations and comments, have been created. In addition, the main table is divided into four categories, each of them having several sub-categories.

How to complete the table(s)? The first table entitled general equations contains the main equations used in the codes to calculate mean velocity, pollutants concentration, and turbulent kinetic energy and its dissipation. These equations have been numbered so that they may be referred to in the main table. Any additional terms in the general equations should be put in this table and numbered. However, additional terms that are not general should be added in the code description table directly, e.g. equation for the production of pollutants or for traffic induced turbulence etc.

The code description table is the main table. It is divided into four categories:

- Equations. In this section the corresponding number of the main equations used in the code is required plus any extra non-general terms.

- Constants. The value of the constants used by the equations is added here.

- Numerical scheme. The order of the scheme as well as the ways that the differential equations are calculated should be added in this section.

- Boundary conditions. The conditions at the inflow, outflow, top of the domain, and ground for velocity, turbulence, pressure and pollutants concentration are given here.

Further information and comments for each of these categories, e.g. description of the way that the boundary conditions are prescribed, sketches etc., together with the required nomenclature should be added in the comments table given below the main one.

For any comments on the preparation of the tables contact Petroula.Louka@ec-nantes.fr

# General equations

| Navier-Stokes equations | ∂Ui∂t+Uj∂Ui∂xj=−1ρn∂P∂xi−∂uiuj∂xj | (a) |
| --- | --- | --- |
|  | ∂ρUi∂t+∂ρUjUi∂xj=−∂P∂xi−∂ρuiuj∂xj | (1b) |
|  | ∂Ui∂t+∂UjUi∂xj=−1ρn∂P∂xi−∂uiuj∂xj | (1c) |
| Continuity equation | ∂Uj∂xj=0 | (2a) |
|  | ∂ρ∂t+∂ρUj∂xj=0 | (2b) |
| Reynolds stress | −uiuj=νt∂Ui∂xj+∂Uj∂xi−23δijk | (3a) |
|  | −ρuiuj=μ+μt∂Ui∂xj+∂Uj∂xi | (3b) |
|  | −ρuiuj=μt∂Ui∂xj+∂Uj∂xi−23ρδijk | (3c) |
| The transport equation for pollutants | ∂C∂t+Uj∂C∂xj=−∂ujc∂xj+Spol | (4a) |
|  | ∂ρC∂t+∂ρUjC∂xj=−∂ρujc∂xj+Spol | (4b) |
|  | ∂C∂t+∂UjC∂xj=−∂ujc∂xj+Spol | (4c) |
| Pollutant flux | −ujc=νtSct∂C∂xj | (5) |
| Turbulent kinetic energy | ∂k∂t+Uj∂k∂xj=−uiuj∂Ui∂xj+∂∂xjνtσk∂k∂xj−ε | (6a) |
|  | ∂ρk∂t+∂ρUjk∂xj=−ρuiuj∂Ui∂xj+∂∂xjμ+μtσk∂k∂xj−ρε | (6b) |
|  | ∂ρk∂t+∂ρUjk∂xj=∂∂xjρμtσk−1∂k∂xj+τij∂Ui∂xj−ρε | (6c) |
|  | ∂k∂t+∂Ujk∂xj=−uiuj∂Ui∂xj+∂∂xjνtσk∂k∂xj−ε | (6d) |
| Dissipation | ∂ε∂t+Uj∂ε∂xj=−Cε1εkuiuj∂Ui∂xj+∂∂xjνtσε∂ε∂xj−Cε2ε2k | (7a) |
|  | ∂ρε∂t+∂ρUjε∂xj=−Cε1εkρuiuj∂Ui∂xj+∂∂xjμtσε∂ε∂xj−Cε2ρε2k | (7b) |
|  | ∂ε∂t+∂Ujε∂xj=−Cε1εkuiuj∂Ui∂xj+∂∂xjνtσε∂ε∂xj−Cε2ε2k | (7c) |

# Code description

| EQUATIONS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CHENSI-1 Standard k- 3-D Unsteady in time | CHENSI-2 k- models (standard; Chen & Kim, 1987; RNG) 3-D Unsteady in time | MIMO 0-,1-,2-equation turbulence models; Standard k-, various linear and non-linear k-, k-ω | MISKAM Standard k- 3-D | TASCflow |
| Mean velocity |  | (1a), (2a) | (1c), (2a) | (1b),(2b) | (1a) buoyancy omitted, (2a) | (1b), (2b) |
| Reynolds stress |  | (3a) | (3a) | (3c) | (3a) last term assumed part of pressure perturbation | (3b) |
| Transport of pollutants |  | (4a) | (4c) | (4b) | (4a) | (4b) |
| Concentration flux |  | (5) | (5) | (5) | (5) | (5) |
| Production of pollutants |  | Only in the source cells: | Only in the source cells: Volume source Q [kg/s] | Only in the source cells: Volume source Q [kg/s] | Only in the source cells: Volume source Q [g/s] | Only in the source cells: Volume source Q [g/s] |
| k |  | (6a) | (6d) | (6c) | (6a) | (6b) |
|  |  | (7a) | (7c) | (7b) | (7a) | (7b) |
| CONSTANTS |  |  |  |  |  |  |
|  |  | CHENSI-1 | CHENSI-2 | MIMO | MISKAM | TASCflow |
| C |  | 0.09 | 0.09 | 0.09 | 0.09 | 0.09 |
| C1 |  | 1.44 | 1.44 | 1.44 | 1.44 | 1.44 |
| C2 |  | 1.92 | 1.92 | 1.92 | 1.92 | 1.92 |
| k |  | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 1.3 | 1.3 | 1.3 | 1.3 | 1.3 |
| Sct |  | 0.90 | 0.74 | 0.74 | 0.74 | 0.75 |
|  |  | 0.40 | 0.40 | 0.43 | 0.40 | 0.41 |
| R0 |  | 287.0 |  | 287.04 |  |  |
| NUMERICAL SCHEME |  |  |  |  |  |  |
|  |  | CHENSI-1 | CHENSI-2 | MIMO | MISKAM | TASCflow |
| Order in time |  | 1st explicit | 1st explicit | 2nd explicit | 1st explicit | 1st or 2nd Implicit |
| Order in space |  | Between 1st & 2nd | 1stor 2nd | 2nd | 1st | 2nd |
| Convection terms |  | Upstream weighted | Hybrid or HLPA (Hybrid Linear Parabolic Approximation) scheme | 2nd order total-variation-diminishing (TVD) or flux-corrected transport (FCT) Adams-Bashforth | Upstream | Skew–upwind with Physical Advection Correction |
| Diffusion terms |  | Centred | Centred | Centred | Centred | Centred |
| BOUNDARY CONDITIONS |  |  |  |  |  |  |
|  |  | CHENSI-1 | CHENSI-2 | MIMO | MISKAM | TASCflow |
|  | U,V, W | Dependent of the inflow information read from input file | Dependent of the inflow information read from input file | Dependent of the inflow information read from input file | Ideal inflow profile assumed according to the given parameters z0, Href, U(Href) | Dependent on the inflow information read from input file |
|  | k |  |  |  |  |  |
| Inflow |  |  |  |  |  |  |
|  | P |  |  |  |  |  |
|  | C |  |  |  |  |  |
|  | U,V, W | Zero normal first derivative | Zero normal first derivative | Zero normal first derivative | Zero normal first derivative | Zero normal first derivative |
|  | k |  |  |  |  |  |
| Outflow |  |  |  |  |  |  |
|  | P |  |  | Pressure correction to ensure overall mass conservation | Pressure correction to ensure overall mass conservation |  |
|  | C |  |  | Zero normal first derivative | Zero normal first derivative |  |
|  | U, V, |  | Zero normal first derivative or boundary layer condition | Dirichlet, staggered Dirichlet (zero normal velocity) or zero normal first derivative | U=const., V=const., W=0 | Symmetry boundary conditions applied (zero gradients and zero normal velocity). A general “Opening” boundary condition can also be applied. |
|  | W |  | Zero normal first derivative or zero normal second derivative or boundary layer condition | Zero flux |  |  |
| Top of domain | k | Zero normal first derivative | Zero normal first derivative or zero normal second derivative or boundary layer condition | Staggered Dirichlet (zero flux) or zero normal first derivative | k=const. |  |
|  |  |  | Zero normal first derivative or zero normal second derivative or boundary layer condition | Staggered Dirichlet (zero flux) or zero normal first derivative |  = const. |  |
|  | P |  |  | zero normal first derivative | Zero normal first derivative |  |
|  | C | Zero normal first derivative | Zero normal second derivative | Staggered Dirichlet (zero flux) or zero normal first derivative | Zero normal first derivative |  |
|  | U,V, W | Law of the wall | Law of the wall | Law of the wall | No-slip, U=V=W=0 | Law of the wall |
|  | k |  |  |  | k=u2cm |  |
| Walls |  |  |  |  | e=u3k⋅zs |  |
|  | P |  |  |  |  |  |
|  | C | Zero normal first derivative, or zero normal second derivative | Zero flux |  | Zero normal first derivative | Zero flux |

# Comments

| NUMERICAL |  |  |  |  |
| --- | --- | --- | --- | --- |
| CHENSI-1 | CHENSI-2 | MIMO | MISKAM | TASCflow |
| Staggered grid (Arakawa-C-type). U, V, W are defined at the centre of the corresponding face of the cell, while all the other quantities (Scal(I,J,K)) are defined at the centre of the cel | Staggered grid (Arakawa-C-type). U, V, W are defined at the centre of the corresponding face of the cell, while all the other quantities (Scal(I,J,K)) are defined at the centre of the cell | Staggered grid (Arakawa-C-type). U, V, W are defined at the centre of the corresponding face of the cell, while all the other quantities (Scal(I,J,K)) are defined at the centre of the cell | Staggered grid (Arakawa-C-type). U, V, W are defined at the centre of the corresponding face of the cell, while all the other quantities (Scal(I,J,K)) are defined at the centre of the cell | All variables stored at the grid nodes. Control volume is build around each node by octants of the surrounding elements. |
| Definition of convection terms (e.g. ): WDUDZ(I) = 0.5W(I) {(1-SIGN(ALF,W)) [(U(I,J,K+1)-U(I,J,K))/Z(K+1)]+ (1+SIGN(ALF,W)) [(U(I,J,K)-U(I,J,K-1))/Z(K)]} | If φD−2φU+φUU<φD−φUU then φf=φU+φD−φUφU−φUUφD−φUU else φf=φU |  |  |  |
| Definition of diffusion terms (e.g. ): |  |  |  | Diffusion terms computed on each integration point through first derivative: ¶f¶x¦ip=n¶Nn¶x¦ipfn ¶N¶x¶N¶y¶N¶z=¶x¶s¶y¶s¶z¶s¶x¶t¶y¶t¶z¶t¶x¶u¶y¶u¶z¶u−1¶N¶s¶N¶t¶N¶u N1=18(1−s)(1−t)(1−u)N2=18(1+s)(1−t)(1−u)N3=18(1−s)(1+t)(1−u)N4=18(1+s)(1+t)(1−u)N5=18(1−s)(1−t)(1+u)N6=18(1+s)(1−t)(1+u)N7=18(1−s)(1+t)(1+u)N8=18(1+s)(1+t)(1+u) |
| Second order derivatives ¶2U/¶Z2: | Not needed in a Control Volume approach |  |  | Not needed in a Control Volume approach |
| BOUNDARY CONDITIONS |  |  |  |  |
| CHENSI-1 | CHENSI-2 | MIMO | MISKAM | TASCflow |
| Law of the wall for the ground: k1=k2 | τx=Cμ14kPUPκlndPz0 τy=Cμ14kPVPκlndPz0 dkdn)wall=0 PkP=1dP+(τxUP++τyVP+−Cμ34kP32κ−1lndP+z0) εP=Cμ34kP32κzP | 2-zonal wall function: |  | u+=n+,n+≤5u+=d1n+3+d2n+2+d3n++d4,5≤n+≤30u+=1kln(n+)+C,n+³30u+=utut n+=ρDnutmsometimes denoted as y+ ut=twρ1/2 |
| Zero normal first derivative e.g. for the concentration: |  |  |  |  |
|  | Boundary layer condition ∂U∂z=Cμ14ktopκztopUtopUtop2+Vtop2 ∂V∂z=Cμ14ktopκztopVtopUtop2+Vtop2 ∂2W∂z2=0 ∂k∂z=0 ∂ε∂z=−Cμ34ktop32κz2 |  |  |  |
| NOMENCLATURE |  |  |  |  |
| CHENSI-1 | CHENSI-2 | MIMO | MISKAM | TASCflow |
| ALF = 0.8 | Subscripts used : - for convection term : f for face of the control volume D for downstream node U for upstream node UU for remote upstream node - for law of the wall : P for computation node P+ for grid line “above” (opposite to the wall) node P |  | zs : distance to the wall | P=P+23dijρ |
| SOURCE in g m-1 s-1 | d : distance between a point and a wall |  |  | w: wall shear stress |
| z0walls: Roughness of walls |  |  |  | ut: known velocity tangent to wall |
| A: Function of X, (or Y or Z), z0wall, k and t. |  |  |  | n: distance from the wall |
| B: Function of Z, z0, k and t. |  |  |  | C=5.2: constant depending on the wall-roughness. For rough walls C in the log law is replaced by Crough=C−1kln1+0.3nr+ |
|  |  |  |  | nr+=ρunr/m, nr=30z0 |
|  |  |  |  | The near wall production is: Pk=tvisc2mPk+ |
|  |  |  |  | Pk+=n+u+2du+dn+ |

e-mails

| CHENSI-1 | Petroula.Louka@ec-nantes.fr or Jean-Francois.Sini@ec-nantes.fr |
| --- | --- |
| CHENSI-2 | Emmanuel.Guilloteau@univ-valenciennes.fr |
| MIMO | peter@aix.meng.auth.gr or moussio@vergina.eng.auth.gr |
| MISKAM | MKE@DMU.dk or joachim.eichhorn@uni-mainz.de |
| TASCflow | peter@aix.meng.auth.gr or moussio@vergina.eng.auth.gr |